LAB 8 - IMPLEMENT VLANS AND TRUNKING

Course Code - Course Name: - COMP4039 – Network Foundations

Program:  T433 - Cybersecurity

Section: A

Term: - Winter 2024

Group Number: 06

Student Names - ID:

- Prabhjot Singh Sains – 101495218

- Rahul Patel - 101378458

- Jai Deep Rawat - 101503760

- M. Salmaan Mustafa Shah – 10151007

Lab Report by - Prabhjot Singh Sains

Topology

Addressing table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 10 | 192.168.10.11 | 255.255.255.0 |
|  | VLAN 20 | 192.168.20.11 | 255.255.255.0 |
|  | VLAN 30 | 192.168.30.11 | 255.255.255.0 |
| S2 | VLAN 10 | 192.168.10.12 | 255.255.255.0 |
| PC-A | NIC | 192.168.20.13 | 255.255.255.0 |
| PC-B | NIC | 192.168.30.13 | 255.255.255.0 |

VLAN Table

| VLAN | NAME | Interface assigned |
| --- | --- | --- |
| 10 | Management | S1: VLAN 10 S2: VLAN 10 |
| 20 | Sales | S1: VLAN 20 and F0/6 |
| 30 | Operation | S1: VLAN 30 S2: F0/18 |
| 999 | ParkingLot | S1: F0/2-5, F0/7-24, G0/1-2 S2: F0/2-17, F0/19-24, G0/1-2 |
| 1000 | Native | N/A |

Instructions

Part A: Set up the topology and initialize the devices:

- Cable the network as shown in the topology diagram and turn on your devices.

- Connect the rollover console cable to the RJ-45 console port of the switches S1 and S2 and start any available emulation program such as (Tera Term, putty, or Hyper Terminal)

- Initialize and reload the switches to clear any existing configuration using the following commands:

Switch> enable

Switch# delete vlan.dat

Switch# erase startup-config

Switch# reload

Fig 1 - S1

Fig 2 – S2

Part B: Configure basic configuration and verify connectivity:

- Console into the switch and enable privileged EXEC mode: use the enable command then use the configure terminal command to navigate to global configuration mode.

Fig 3 - S1

Fig 4 – S2

- Use the hostname command to change the switches' names to S1 and S2. The commands provided as a sample for S1, you should do the same for S2.

switch (config)# hostname S1

S1(config)#

Fig 5 - S1

Fig 6 – S2

- Disable DNS lookup to prevent unwanted DNS lookups.

S1(config)# no ip domain-lookup

S2(config)# no ip domain-lookup

Fig 7 - S1

Fig 8 – S2

- Assign the class as the privileged EXEC encrypted password.

S1(config)# enable secret class

S2(config)# enable secret class

Fig 9 - S1

Fig 10 – S2

- Assign Cisco as the console password and enable login.

S1(config)# line con 0

S1(config-line)# password cisco

S1(config-line)# login

S1(config-line)# exit

S1(config)#

Fig 11 – S1

Fig 12 – S2

- Assign Cisco as the VTY password and enable login.

S1(config)# line vty 0 15

S1(config-line)# password cisco

S1(config-line)# login

S1(config-line)# exit

S1(config)#

Fig 13 – S1

Fig 14– S2

- Encrypt the plaintext passwords.

S1(config)# service password-encryption

Fig 15 – S1

Fig 16 – S2

- Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

S1(config)# banner motd # Unauthorized access is strictly prohibited. #

S1(config)# exit

S1#

Fig 17 – S1

Fig 18 – S2

- Save the configuration. Use the copy command to save the running configuration to the startup file on non-volatile random-access memory (NVRAM).

R# copy running-config startup-config

Fig 19 – S1

Fig 20 – S2

- Configure PC hosts: Refer to the Addressing Table for PC host address information

Fig 21 – PC-A

Fig 22 – PC-B

- Configure an IP address on both switches for SVI (VLAN 1), refer to the addressing table.

S1(config)# interface vlan 1

S1(config-if)# ip address 192.168.1.1 255.255.255.0

S1(config-if)# no shutdown

S1(config-if)# exit

S1(config)# exit

Fig 23 – S1

Fig 24 – S2

- Test the connectivity between PCA and PCB, and troubleshoot as necessary.

Use the command prompt (cmd.exe) window to verify the connectivity between PCs.

From PC-A Type >ping 192.168.2.3 and press Enter. Were the ping results successful? No

Fig 25 – PC-A

From PC-B Type >ping 192.168.1.3 and press Enter. Were the ping results successful? No

Fig 26 – PC-B

Fig 27 – Topology

Part C: Create VLANs and Assign Switch Ports:

- Create and name the required VLANs on each switch from the table above. Use the VLAN command.

Fig 28 – S1

Fig 29– S2

- Configure the management interface on each switch using the IP address information in the Addressing Table.

Fig 30 – S1

Fig 31 – S2

- Assign all unused ports on the switch to the ParkingLot VLAN, configure them for static access mode, and administratively deactivate them.

Fig 31 – S1

Fig 32 – S1

Fig 33 – S2

Fig 34 – S2

- Assign used ports to the appropriate VLAN (specified in the VLAN table above) and configure them for static access mode.

Fig 35 – S1

Fig 36 – S2

- Verify that the VLANs are assigned to the correct interfaces.

Fig 37 – S1

Fig 38 – S2

- Manually configure trunk interface F0/1 on both switches.

Fig 39 – S1

Fig 40 – S2

- Set the native VLAN to 1000 on both switches.

Fig 41 – S1

Fig 42 – S2

- specify that only VLANs 10, 20, 30, and 1000 are allowed to cross the trunk.

Fig 43 – S1

Fig 44 – S2

- Issue the show interfaces trunk command to verify trunking ports, the native VLAN, and allowed VLANs across the trunk.

Fig 45 – S1

Fig 46 – S2

- Verify connectivity within a VLAN. For example, PC-A should be able to ping S1 VLAN 20 successfully.

Fig 47 –Ping